INSTRUCTIONS:

PLANOS PUERTA.SWP is identical to PLANOS.SWP, except that it adds a view that hides any element with a name containing "PUERTA" or "DOOR". If the element is a sub-assembly, all its contents will be hidden.

These macros operate on PARENT assemblies containing CHILD components that correspond to each module. They do not work with standalone modules.

The names of the PARENT assembly and the first-level CHILD components must not contain spaces.

1-	Always run the macro from `Sub main()`, not from `ReconstruirCuadroYPlanos` or `LimpiarPlanosAuxiliares`.

2-	Set the working language: 0 = Spanish; 1 = English.

Const IDIOMA As Integer = 1   ' <-- 0 = Spanish, 1 = English

3-	If you are a SWOOD user, change this setting to TRUE.

Const SWOOD_INSTALADO As Boolean = False   ' <-- ADJUST according to the computer

4-	OFFSET_CUADRO controls how far the section lines extend beyond the actual outline of the furniture in the view (default is 200mm).

Const OFFSET_CUADRO As Double = 0.2   ' 200 mm

5-	Adjust the view's distance from the top and left sheet margins:

Const MARGEN_SUPERIOR As Double = 0.02   ' 2 cm

Const MARGEN_LATERAL As Double = 0.04    ' 2 cm